Purpose

The purpose of the CANable is to be able to communicate between the JETSON and the motor controllers of the drive wheels using CANbus communication protocol. This off-the-shelf board facilitates this communication. (idk I wrote this fast)

CANable Model: MKS CANable V2.0

Important Links:

Github: https://github.com/makerbase-mks/CANable-MKS/tree/main

CANable Website: https://canable.io/

Python-can library documentation: https://python-can.readthedocs.io/en/stable/bus.html#transmitting

Cangaroo software (not mandatory, used to see sent and received messages): https://canable.io/getting-started.html#cangaroo

CANable Firmware Update:

Follow the following link to be able to update the board.

https://www.youtube.com/watch?v=6MChPbeG6D0&ab_channel=MakerbaseTeam

Note 1: You can choose to update on the website or by downloading a software. The former is preferred because of its simplicity.

Note 2: Contrary to the video, please select “slcan” instead of “candlelight”. It was chosen because of its compatibility with Windows. Linux works well with “candlelight” but if the user decides to use this instead, the code needs to be modified accordingly.

Note 3: Before updating the firmware, please first move the jumper on the board from 120R to BOOT. Switch back to 120R when you are done. (120R is for the termination resistor value which allows for CAN communication.)

Set Up:

- Connect H and L (corresponding to CANL and CANH) on the screw terminal to the corresponding CAN High and CAN Low of the board you are connecting to. Do not forget to connect the GND pin as well.

- Run the Python code.

  - If it is not properly sending, you can use cangaroo to see if the CAN messages are sending at all. [FINISH, how to set up cangaroo]